Installation of Alpha MonitorMouse for Windows NT Driver

- Insert the alpha release diskette into the floppy drive.

- Choose Start/Settings/Control Panel and double-click the Mouse object.

- Select the General tab and choose the Change button.

- Choose Have Disk and choose the floppy drive for the alpha release diskette.

- Choose "Elo Touchscreen SmartSet serial controller on COM2" and choose OK. If the Windows NT CD is requested for mouclass.sys, choose browse, then Skip the file.

- Before restarting the machine you will need to make manual modifications to the Windows NT Registry for the correct serial port. The default is "2" for COM2. Change the number of the serial port in the following registry key by running Start/Run/REGEDIT:

HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\Monmouse\Touchscreen0

\Parameters\ComPort

- Restart the computer. The touchscreen should now work, but is calibrated in the next step.

- Run the calibration program, elocal32.exe, from the \Program Files\Elo directory.

Changing registry entries

Change other values with REGEDIT as necessary. For example, change the following key from 0 to 1 if you want beeps on touch.

HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\Monmouse\BeepOnOff

Restart the computer.