SN24/7-97

DR655xx

Display Driver Package

Release Notes for Windows NT 4.00

Revision:	Version 1.0.8
Release Date:	May 22, 1997

The following enhancements have been made:

- Added support for NTSC/PAL CRT Panning.

- Added support for 65555 and 68554 chips.

The following problems have been fixed in the current release:

- Pre-install of driver does not work.  Defaults to VGA.

- 65548 - Cannot switch to Full Screen DOS session.

- 65548 - When in Display Settings, selecting Test to test new display setting causes system to hang.

Failures 1 and 2 are due to a problem with BIOS Set Refresh function, which was introduced in later versions of 65548 Video BIOS.  Fix has been temporarily made in miniport driver until BIOS fix is available.

- 65545/548 VL/ISA bus systems - The system will hang when doing a Restart after shutdown.

- When switching to NTSC video, two cursor images appear on screen.

- Cursor movement and position is not correct when Graphics Mode stretching is enabled.  Switch to software cursor is now done to allow proper cursor movement.

- Fixed NT 4.0 J problem switching to DOS fullscreen session.

- Underline Text Cursor is missing after switching from Desktop to DOS Command Prompt fullscreen session.

- Text Expansion gets reset during restore of DOS Command Prompt fullscreen session.

- Invalid refresh selection (0 Hz) listed in Display Settings.

- Fixed problem with corruption in display window when using Media Player to play and MPEG (.MPG) file.

- Fixed problem with AVI window freezing when playing an overlapping DOS Command Prompt window was switched to fullscreen, then switched back to a window, and then minimized to tak bar.  Media Player control would indicate that the video clip was playing, but nothing was being drawn in AVI window.

- Fixed problem with 24bpp Transparent SRC/DST bitblt operations in Direct Draw.

- Modified code to correctly output large font (glyphs) that reside in “packed” font files.

- Full screen graphic VDMs are stretched vertically and horizontally.

- Mouse cursor disappears when you go to the right edge or bottom edge in 800x600 or 1024x768 modes.

- Start WinNT 4.0 with true color mode (mode 50 or 52).  Select any background pattern from the Display Properties Background sheet.  Click OK.  You will see that the background palette is corrupt.

- Memory size detection incorrect on 65554 which caused display modes to be excluded from selection.